CPE301 – SPRING 2019

# Design Assignment 5

Student Name: Mat Tomljenovic, Partner: Eric Smith

Student #: 1012254710

Student Email: tomljeno@unlv.nevada.edu

Primary Github address: https://github.com/matcroatia/

Directory: https://github.com/matcroatia/submission_da

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmega 328P microcontroller, NRF24L01 radio frequency transmitter, LM34 temperature sensor.

- INITIAL/MODIFIED/DEVELOPED CODE OF TASK 1/A

/*

* DA5.c

*

* Created: 4/25/2019 10:21:48 PM

* Author : Mat Tomljenovic

*/

#ifndef F_CPU

#define F_CPU 16000000UL

#endif

#include <avr/io.h>

#include <util/delay.h>

#include <avr/interrupt.h>

#include <stdbool.h>

#include <stdio.h>

#include <string.h>

//	Set up UART for printf();

#ifndef BAUD

#define BAUD 9600

#endif

#include "inc/STDIO_UART.h"

//	Include nRF24L01+ library

#include "inc/nrf24l01.h"

#include "inc/nrf24l01-mnemonics.h"

#include "inc/spi.h"

void print_config(void);

void read_adc(void);        // Function Declarations

void adc_init(void);

volatile unsigned int adc_temp;

char outs[20];

//	Used in IRQ ISR

volatile bool message_received = false;

volatile bool status = false;

int main(void)

{

//	Set message to send (message cannot exceed 32 characters)

char tx_message[32];				// Define string array

strcpy(tx_message,"hello i am cody");	// Copy string into array

//	Initialize UART

uart_init();

adc_init();

//	Initialize nRF24L01+ and print configuration info

nrf24_init();

print_config();

//	Start listening to incoming messages

nrf24_start_listening();

while (1)

{

read_adc();

adc_temp = (adc_temp*500)/1023 + 20;

snprintf(outs,sizeof(outs),"%3d\r\n", adc_temp);  // print it

strcpy(tx_message,outs);	// Copy string into array

nrf24_send_message(tx_message);

_delay_ms(1500);

if (message_received)

{

//	Message received, print it

message_received = false;

printf("Received message: %s\n",nrf24_read_message());

//	Send message as response

_delay_ms(500);

status = nrf24_send_message(tx_message);

if (status == true) printf("Message sent successfully\n");

}

}

}

//	Interrupt on IRQ pin

ISR(INT0_vect)

{

message_received = true;

}

void print_config(void)

{

uint8_t data;

printf("Startup successful\n\n nRF24L01+ configured as:\n");

printf("-------------------------------------------\n");

nrf24_read(CONFIG,&data,1);

printf("CONFIG		0x%02X\n",data);

nrf24_read(EN_AA,&data,1);

printf("EN_AA			0x%02X\n",data);

nrf24_read(EN_RXADDR,&data,1);

printf("EN_RXADDR		0x%02X\n",data);

nrf24_read(SETUP_RETR,&data,1);

printf("SETUP_RETR		0x%02X\n",data);

nrf24_read(RF_CH,&data,1);

printf("RF_CH			0x%02X\n",data);

nrf24_read(RF_SETUP,&data,1);

printf("RF_SETUP		0x%02X\n",data);

nrf24_read(STATUS,&data,1);

printf("STATUS		0x%02X\n",data);

nrf24_read(FEATURE,&data,1);

printf("FEATURE		0x%02X\n",data);

printf("-------------------------------------------\n\n");

}

/* INIT ADC */

void adc_init(void)

{

/** Setup and enable ADC **/

ADMUX = (0<<REFS1)|    // Reference Selection Bits

(1<<REFS0)|    // AVcc - external cap at AREF

(0<<ADLAR)|    // ADC Left Adjust Result

(1<<MUX2)|     // Analog Channel Selection Bits

(0<<MUX1)|     // ADC4 (PC4 PIN27)

(0<<MUX0);

ADCSRA = (1<<ADEN)|    // ADC ENable

(0<<ADSC)|     // ADC Start Conversion

(0<<ADATE)|    // ADC Auto Trigger Enable

(0<<ADIF)|     // ADC Interrupt Flag

(0<<ADIE)|     // ADC Interrupt Enable

(1<<ADPS2)|    // ADC Prescaler Select Bits

(0<<ADPS1)|

(1<<ADPS0);

}

/* READ ADC PINS */

void read_adc(void)

{

unsigned char i = 4;

adc_temp = 0;

while (i--)

{

ADCSRA |= (1<<ADSC);

while(ADCSRA & (1<<ADSC));

adc_temp+= ADC;

_delay_ms(50);

}

adc_temp = adc_temp / 4;  // Average a few samples

}

- DEVELOPED MODIFIED CODE OF TASK 2/A from TASK 1/A

Same as above.

- SCHEMATICS

Use fritzing.org

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- VIDEO LINKS OF EACH DEMO

https://youtu.be/8FSChl0G12U

- GITHUB LINK OF THIS DA

https://github.com/matcroatia/submission_da

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Mat Tomljenovic